DISASSEMBLY AND REPAIR

Product: ATLAS AIR

| Replaceable parts | Tools needed |
| --- | --- |
| Armrests | Screwdriver |
| Seat/swivel base | Screwdriver |
| Seat attachment | Screwdriver |
| Starbase | Rubber mallet |
| Swivel column, cover tube | Screwdriver, rubber mallet |
| Wheels/glides | - |

Instructions for replacement

Armrests	Loosen both screws and remove the armrest. Put the new armrest in place and reattach it with screws.

Seat/	Loosen all screws in the seat attachment and remove the swivel base. Replace the seat and

swivel base	fasten the base (with armrests: add the steps above).

Seat	Loosen all screws and remove the seat attachment. Ask a friend for assistance. One person
attachment	takes a firm grip around the pillar while the other uses a rubber mallet to remove the seat 	attachment from the pillar by striking it. Place the new attachment on top of the pillar and 	push down firmly, with hands and body weight. Place the seat upside down with the entire 	base on top (with the height adjustment lever on the left side). Reattach the seat attachment 	with screws. Check that all parts are connected.

Starbase	Ask a friend for assistance. One person takes a firm grip around the pillar while the other uses a rubber mallet to remove the starbase from the pillar by striking it. Detach the wheels/glides from the old base and fasten them on the new one. Place the chair on top of the new base and push down firmly, with hands and body weight. Check that all parts are connected.

Swivel	Ask a friend for assistance. One person takes a firm grip around the seat while the other 
column, 	uses a rubber mallet to remove the seat attachment from the pillar by striking it. 
cover tube	To switch the cover tube: Replace the cover tube. Put the swivel base on top and strike it 	with a rubber mallet. Check that all parts are connected.
	To switch the swivel column: Repeat the first step but hold on to the pillar instead. Put the 	seat upside down. Place the column in the seat attachment with the smaller side upwards. 	Put the cover tube in place and the swivel base on top. strike it with a rubber mallet. Check 	that all parts are connected.

Wheels,	Turn the chair upside down remove the glide or wheel out of the starbase by pulling it. 
glides	Replace it and push down firmly.

Disassembly for recycling

Loosen all screws with a screwdriver to remove the seat, backrest top and armrests. Use a knife or a pair of scissors to cut off the mesh fabric from the frame. Use a rubber mallet to disassembly the base. Remove the glides/wheels by pulling them.