DISASSEMBLY AND REPAIR

Product: CAMPUS AIR BARSTOOL

| Replaceable parts | Tools needed |
| --- | --- |
| Glides | Adjustable pliers, rubber mallet |
| Seat | Screwdriver |

Instructions for replacement

Glides	Pull off the glide with adjustable pliers. Insert the new glide, position it correct and strike it with a rubber mallet.

Seat	Place the chair upside down on a table and use a screwdriver to remove the screws, 3 in the front and 2 in the back. Remove the old seat and replace it with the new one. Make sure the new seat has all bumpers in place.

Start by tighten the screw placed in the middle front hole, to make sure that the frame is in position. Continue with the remaining screws.

Disassembly for recycling

Place the chair down on the floor and use a screwdriver to remove the seat. To remove the backrest, saw the back in half down the middle and twist the parts to detach them from the frame. Pull off the glide with adjustable pliers.