DISASSEMBLY AND REPAIR

Product: CAMPUS AIR SLED BASE

| Replaceable parts | Tools needed |
| --- | --- |
| Glides | Screwdriver |
| Seat | Screwdriver |

Instructions for replacement

Glides	Place the chair upside down on a table and use a screwdriver to remove the old glides. Put the new ones in place and tighten the screws.

Seat	Place the chair upside down on a table and use a screwdriver to remove the screws, 3 in the front and 2 in the back. Remove the old seat and replace it with the new one. Make sure the plastic distances are in correct position.

Start by tighten the screw placed in the middle front hole, to make sure that the frame is in position. Continue with the remaining screws.

Disassembly for recycling

Place the chair upside down on a table and use a screwdriver to remove the seat. Pull the stacking bumpers off by hand. To remove the backrest, saw the back in half down the middle and twist the parts to detach them from the frame. Pull off the glide with adjustable pliers. To remove the armrests, use a screwdriver to loosen the screws and strike the armrest with a rubber mallet to separate it from the frame.